BVVS Sangha’s

Basaveshwara Engineering College, Bagalkote

DEPARTMENT OF ELECTRONICS AND COMMUNICATION  ENGINEERING

Major Project on

AI based ECG signal analysis and disease prediction using Raspberry pi

| SL.NO | NAME | USN | ROLL NO | DIVISION |
| --- | --- | --- | --- | --- |
| 1 | Rahul.Kapaleshwar | 2BA22EC074 | 06 |  |
| 2 | Souparni.Kulkarni | 2BA22EC107 | 38 | B |
| 3 | Spoorti.Deshpande | 2BA22EC108 | 39 |  |
| 4 | Suhas.Kulkarni | 2BA22EC112 | 43 |  |

Under Guidance of                                     HoD                                       Project Coordinator

Dr. Vijaylakshmi Jigajinni         Dr. Jayashree.D. Mallapur                     Dr. Sharanappa. P. H.

Prof. Supriya. B. H

Abstract:

The system acquires electrocardiogram (ECG) and Peripheral Capillary Oxygen Saturation (SpO2) signals through the MAX98101 sensor and ECG electrodes interfaced with Raspberry Pi’s ADC (via an external ADC module MCP3008). The Raspberry Pi performs digital signal processing (filtering, normalization, and R-peak detection) using MATLAB for signal processing and Python-based algorithms. Features such as R-peak amplitude, RR interval, and speed, along with patient metadata (age, gender), are extracted.

These features are passed to a machine learning classifier (decision tree) trained and validated on labelled  ECG datasets. The trained model categorizes cardiac conditions as Normal, Bradycardia, Tachycardia, or Ventricular Tachycardia. The Raspberry Pi then displays the results (pulse rate + prediction) on an OLED display in real time.

This standalone Raspberry Pi system eliminates the need for a PC backend, enabling compact, low-cost, and intelligent cardiac monitoring, with potential applications in IoT healthcare and telemedicine.

Introduction:

Cardiovascular diseases demand efficient and continuous monitoring for early diagnosis. The ECG is one of the most effective diagnostic tools for detecting arrhythmias and other cardiac abnormalities. Traditional ECG systems are often expensive and hospital-dependent, limiting their availability for personal health monitoring.

This project proposes a Raspberry Pi–based intelligent ECG monitoring system that integrates sensor-based data acquisition, digital signal processing, and machine learning classification. Unlike microcontroller-only solutions, Raspberry Pi provides sufficient computational power to handle data acquisition, processing, and classification locally in real time. The results, including pulse rate and diagnostic condition, are displayed on an OLED screen, making the system compact, portable, and intelligent.

Objectives:

- To develop a portable, cost-effective ECG monitoring system using a Raspberry Pi, ECG sensor MAX98101, and an OLED display or monitor for real-time heart monitoring.

- Acquire ECG signals in real time with the Raspberry Pi and transmit the signals to a connected system or software over Wi-Fi to MATLAB for advanced digital processing.

- The AI algorithm used is a Decision Tree Classifier, while the signal processing stage includes noise filtering, normalization, and R-peak detection to ensure accurate ECG feature extraction and interpretation.

- Extract key cardiac features like R-peak amplitude, RR intervals, along with patient details (age, gender, activity level) to build a comprehensive dataset for health analysis.

Expected Outcomes:

- A functional portable ECG monitoring prototype built around Raspberry Pi with live ECG visualization and feature extraction.

- Real-time data handling and transmission achieved via Raspberry Pi’s GPIO, SPI, or Wi-Fi capabilities, supporting seamless signal acquisition and processing integration.

- Robust signal processing implemented, resulting in clean ECG signals with reliable R-peak detection and noise suppression.

- Intuitive and immediate display of diagnostic results on an OLED screen or monitor connected to the Raspberry Pi, enhancing usability.